AAS 19-612

Assessing GEO and LEO Repeating Conjunctions using High Fidelity Brute Force Monte Carlo Simulations

Luis Baars, Doyle Hall, and Steve Casali

Probability of collision (Pc) estimates for Earth-orbiting satellites typically assume a temporally-isolated conjunction event. However, under certain conditions two objects may experience multiple high-risk close approach events over the course of hours or days. In these repeating conjunction cases, the Pc accumulates as each successive encounter occurs. The NASA Conjunction Assessment Risk Analysis team has updated its “brute force Monte Carlo” (BFMC) software to estimate such accumulating Pc values for repeating conjunctions. This study describes the updated BFMC algorithm and discusses the implications for conjunction risk assessment.

Introduction

The NASA Conjunction Assessment Risk Analysis (CARA) team estimates the probability of collision (Pc) for a specific set of high value, Earth-orbiting satellites. The CARA processing system first detects candidate close encounters involving high value assets up to ten days in advance using a screening-volume approach, based on the latest available satellite tracking data and orbit determination (OD) state and covariance solutions.1,2

For each conjunction, CARA assesses the collision risk using a set of established semi-analytical, and Monte Carlo Pc approximation methods.3,4,5 Under certain circumstances, a satellite may experience multiple close approach events with another object over the course of hours or even days. For these “repeating” conjunctions, collision probability accumulates as each close approach event occurs. Many GEO satellites orbit within clusters (by mission design), so it follows that these closely-spaced objects experience a higher frequency of repeating conjunctions. GEO cluster objects can also experience long duration encounters due to their low relative velocities.4 Repeating and long-duration encounters also occur at other orbit regimes.6 Most widely-used semi-analytical Pc estimation methods only consider temporally-isolated, short-duration conjunctions.4,5,7 One approach for assessing collision risk for repeating conjunctions would be to combine semi-analytical, single-encounter Pc estimates of multiple successive encounters in order to estimate a cumulative Pc. However, this approach implicitly assumes that the repeating conjunctions are statistically independent events, and may yield inaccurate results, especially for repeating long-duration encounters that are closely spaced in time and effectively overlap or blend with one another.

CARA requires a Pc estimation method to assess collision risk for repeating conjunctions accurately. In a previous study,7 the CARA team presented a “brute force Monte Carlo” (BFMC) method using Special Perturbations (SP) orbital propagation8 within a Monte Carlo (MC) framework.9 The BFMC algorithm is general enough to account for the accumulation of collision probability during a temporally-isolated conjunction, or throughout a series of repeating and/or long-duration encounters.7 The previous version of the BFMC software, however, was only implemented to address temporally-isolated conjunctions. The updated BFMC implementation differs from the original by increasing the collision risk assessment period from a relatively short interval (that closely brackets the time of peak risk for a temporally-isolated encounter) to an extended duration (that can span multiple encounters).

When analyzing repeating conjunctions, this extended risk assessment period can begin as early as the most recent of the two OD epoch times for the primary and secondary satellites, and end several days later. Because of this, collisions in the MC simulation can be distributed over an extended, multi-orbit interval, much longer than the typical span (measured in seconds to minutes) for temporally-isolated conjunctions. Additionally, the collisions can be distributed in time in one of three characteristic ways:

1. 	They occur in brief “bursts” separated by regular intervals, producing a stair-step trend in cumulative Pc characterized by relatively long durations without a collision.

2. 	The collision bursts widen and blend together such that the Pc accumulates more steadily over the risk assessment interval.

3. 	There is a combination of the stair-step and blended Pc trends.

As will be shown later, a cumulative Pc plot can be used to visualize how risk increases throughout a repeating conjunction interaction, and assess if and when it may exceed an acceptable maximum-risk level for different missions. This paper describes recent updates to the BFMC algorithm in detail, presents cumulative Pc plots for various GEO and LEO repeating conjunction cases, and discusses the implications for future conjunction risk assessments.

The BFMC Algorithm

This section presents a brief overview of the BFMC algorithm to provide context for the recent updates implemented for processing repeating conjunctions.

Original BFMC Algorithm

MC Pc estimation simulations can be computationally intensive because they require repeatedly performing the following steps:7,10

1. 	Sample the orbital state probability density functions (PDFs) of the primary and secondary satellites at some epoch.

2. 	Use the sampled states to propagate satellite state vectors throughout a risk assessment period, determining if the distance between the objects ever becomes less than a combined hard-body protection radius.

3. 	If the miss distance becomes less than the protection radius, register that a simulated collision has occurred.

These three steps together represent one BFMC sampling trial. Trials need to be repeated until enough simulated collisions (or “hits”) have been registered to provide sufficiently accurate statistical results, which may require a large number of samples depending on the conjunction. (For instance, to estimate a collision probability to ~20% accuracy at a 95% confidence level typically requires ~100 hits7 which in turn requires ~106 trials for an event that has Pc = 10-4.) Step 2 requires significant computation, especially for complex propagation schemes7,9 and when assessing collision risks accumulated over extended intervals.

The orbital state sampling in step 1 above can be performed in two distinctly different ways:9

- SP states can be sampled from PDFs estimated at the OD epochs for the primary and secondary satellites, and then propagated forward in time throughout the risk assessment period.

- Alternatively, equinoctial states can be sampled from PDFs predicted for a single conjunction’s nominal time of closest approach (TCA) and propagated forward and backward from that point in time.

These two BFMC approaches are referred to as “from-epoch” and “from-TCA”, respectively.7

For each trial, the BFMC algorithm checks if the distance between the two objects becomes less than a threshold hard-body radius miss distance, H, within the risk assessment interval, and, if so, registers a simulated collision at the corresponding time of first contact. This time of first contact is carefully defined and determined7 such that at most one hit can be registered per trial, thereby avoiding multiple-counting errors that can lead to inflated estimates of risk. BFMC repeats this process for a large number of trials, yielding a best-estimate collision probability of

Pc=Pcτa,τb=Ncτa,τb/Ns	(1)

Here τa and τb denote the beginning and end of the risk assessment interval, Nc the number of hits registered during that interval, and Ns the total number of sampling trials. BFMC also estimates an asymmetric confidence interval for Pc using the Clopper-Pearson method.11,12

In the “from-epoch” mode, BFMC samples SP state vectors at the two OD epochs and propagates them throughout the risk assessment interval. SP state vectors include the six equinoctial orbital elements, supplemented by additional state parameters that account for atmospheric drag and solar radiation pressure orbital perturbations.7-9 BFMC’s “from-epoch” mode uses an SP propagation scheme which incorporates the most recent version of the Jacchia-Bowman atmospheric density model13 plus the associated Dynamic Calibration Atmosphere (DCA) for the High Accuracy Satellite Drag Model (HASDM).14 This scheme was selected specifically to match the currently operational SP software configuration of the Astrodynamics Support Workstation (ASW).15,16 Epochs, states, and uncertainty PDFs are derived from Vector Covariance Messages (VCMs) produced by the ASW OD processing system. Because VCMs for both the primary and secondary object are used as inputs for the processing, BFMC’s “from-epoch” mode is also referred to as “VCM mode.”

The “from-TCA” mode differs from the “from-epoch” mode in two fundamental ways.7 First, it samples equinoctial element state vectors from marginalized PDFs predicted at the conjunction’s nominal TCA, as opposed to the OD epochs. Second, it propagates the orbital states using computationally efficient Keplerian 2-body equations of motion. All input data required for “from-TCA” processing can be derived from a single Conjunction Data Message (CDM), so BFMC’s “from-TCA” mode is also called “CDM mode.” Because the CDM mode employs 2-body propagation, it must be restricted to relatively short risk assessment intervals near the conjunction’s nominal TCA; 2-body propagation neglects orbital perturbations that can affect satellite motion over extended time scales.7

The BFMC Risk Assessment Interval

Coppola17 formulates a method of estimating the effective duration of a single conjunction, which depends on the relative velocity of the encounter as well as the state uncertainties. BFMC utilizes this duration as a basis for determining a risk assessment interval for temporally-isolated conjunctions.7 However, for satellites which experience long-duration and/or repeating conjunctions,6,10,18 the risk assessment interval must be extended. For such cases, BFMC’s VCM-mode has been modified to employ a potentially much longer interval (e.g., extending over multiple orbital periods). This modification results in three distinct BFMC simulation modes:

- CDM mode: “from-TCA” 2-body motion propagations for temporally-isolated conjunctions using short-duration risk assessment intervals based on the Coppola conjunction duration.

- Short-duration VCM mode (SD-VCM): “from-epoch” SP propagations for temporally-isolated conjunctions using short-duration risk assessment intervals based on the Coppola duration.

- Long-duration VCM mode (LD-VCM): “from-epoch” SP propagations for long-duration and/or repeating conjunctions using extended (i.e., multi-orbit) risk assessment intervals.

The software modifications described in this analysis extend the BFMC algorithm by fully implementing the third, LD-VCM mode listed above (the first two had been implemented previously7). When running LD-VCM mode, the registration of simulated collisions at the time of first contact of the two hard-body spheres is the same as SD-VCM mode; in fact, the only difference between these two modes is the length of the risk assessment interval. Great care has been taken to ensure that times of first contact have been determined accurately in all three BFMC modes.

Identifying Repeating Conjunctions

A repeating conjunction occurs when a set of OD solutions produce multiple close approach events between two objects within a multi-orbit period. However, this broad definition could conceivably include a very large number of interactions between Earth orbiting satellites, depending on what cutoff is used to define a “close approach.” This section presents a more rigorous definition of repeating conjunctions, as well as a method to discriminate them from temporally-isolated events.

An SP orbital state is represented by an 8x1 vector, X=n,af,ag, χ,ψ,λM,B,ST, with the first six elements denoting the satellite’s equinoctial orbital elements, and the last two a ballistic coefficient and a solar radiation pressure parameter.7,8,19-21 The SP states can be propagated to predict high fidelity satellite position and velocity vectors at time t, denoted symbolically here as rt;t0,X0,D and vt;t0,X0,D, respectively, where t0 and X0 represent the time and SP orbital state at epoch, respectively, and D represents an ensemble of model and environmental data sets required for SP propagation. Similarly, SP mean states and covariances can be propagated8,19-21 and denoted as X(t;t0,X0,D) and P(t;t0,X0,P0,D), respectively. (Note, in this analysis some or all of the function arguments listed to the right of semicolons may be suppressed for brevity.)

The SP epochs, mean states, and covariances for the primary and secondary objects are denoted as:

t1,0,X1,0,P1,0          and          t2,0,X2,0,P2,0	(2)

respectively.7 (This analysis uses subscripts “1” and “2” to indicate quantities associated with the primary and secondary.) BFMC has been modified to propagate mean SP states and covariance matrices for the primary and secondary objects through an extended interval, τa<t≤τb. For the BFMC LD-VCM mode examples discussed in this study, the beginning and end of the risk assessment interval are selected such that

τa=maxt1,0,t2,0        and           τb=τa+7 days	(3)

However, the BFMC algorithm is not constrained to use these values. For instance, during operational processing, τa could be set to a later time if both epochs are several orbital revolutions old (and also to allow time for risk mitigation maneuver planning and execution). Similarly, τb could be adjusted on a case-by-case basis to span all high-risk close approach events in a repeating sequence, even if some extend beyond a nominal, pre-specified risk assessment interval.

The time-dependent intervening distance between the mean positions for the primary and secondary objects can be written as

r2,1t=r2t;t2,0,X2,0-r1t;t1,0,X1,0	(4)

During a multi-orbit risk assessment interval, r2,1t will vary in time as the objects approach and recede from one another. This means that r2,1t can have multiple local minima, and the times of these can be defined by imposing three conditions:

tlmin= t |  r2,1t=0  and  r2,1t>0  and  τa<t≤τb 	(5)

The first condition requires that the relative velocity, r2,1, is zero; the second uses the relative acceleration, r2,1, to include minima and exclude maxima; the third requires that the minima occur during the risk assessment interval. Each local minimum time in the set tlmin is denoted tlmin,i, with i=1…Nm. BFMC calculates position, velocity, and covariance information at each of the minima for both the primary and secondary objects, denoted here as (r1,i,v1,i,P1,i) and (r2,i,v2,i,P2,i), respectively. These quantities can then be used to calculate a “2D-Pc” semi-analytical collision probability estimate3,22 for each local minimum, idealizing each as a temporally-isolated conjunction.

The current method implemented in BFMC to identify possible repeating conjunctions can be summarized as follows:

1. 	Propagate the mean states and associated covariances of both objects throughout the multi-orbit risk assessment interval τa<t≤τb using high-fidelity SP propagation.

2. 	Find all local minima of separation distance r2,1t throughout the risk assessment interval.

3. 	Calculate a 2D-Pc estimate for each minimum, treating each as an individual conjunction.

4. 	Count the number of these 2D-Pc values which exceed a “repeating conjunction detection threshold” of 10-10 (a level provisionally adopted for current BFMC analyses). If this count exceeds one, then classify the interaction as a repeating conjunction to be processed using BFMC’s LD-VCM mode.

Figure 1: Mean separation distance analysis for a GEO repeating conjunction (top), a LEO repeating conjunction (middle), and a LEO temporally-isolated conjunction (bottom). Red dots indicate single-encounters with 2D-Pc > 10-10 and green dots indicate single-encounters with 2D-Pc  10-10.

Figure 1 plots mean primary-to-secondary separation distances as a function of time over 7-day risk assessment intervals for three archived example conjunctions, in order to provide a visualization of the repeating conjunction identification process implemented in the BFMC system. This section discusses these separation distance curves in detail; subsequent sections discuss and analyze the corresponding BFMC collision probabilities for all three of these examples.

The top panel of Figure 1 shows a multi-orbit interaction between two coplanar GEO-cluster satellites. During this 7-day interval 14 minima in r2,1t occur, at a rate of twice per orbital period. Notably, the 2D-Pc values calculated for each of these minima all exceed the repeating conjunction detection threshold of 10-10. Red dots identify minima with these above-threshold 2D-Pc values, and the occurrence of multiple red dots in the top panel of Figure 1 indicates the detection of a repeating conjunction between these two GEO satellites.

The middle panel of Figure 1 shows an interaction between non-coplanar LEO satellites in which there is a large number of r2,1t minima, seven of which have 2D-Pc values that exceed the repeating conjunction detection threshold. Again, the occurrence of multiple red dots in the plot indicates the detection of a repeating conjunction.

The LEO conjunction plotted in the bottom panel of Figure 1, however, contains only one isolated red dot. This indicates that only one of the minima has a 2D-Pc value that exceeds the repeating conjunction detection threshold. In fact, for this interaction all cumulative collision risk during the entire 7-day interval effectively arises from this single encounter. The majority of conjunctions analyzed by CARA can be considered “temporally-isolated” events such as this, in which all risk effectively accumulates during a single close approach during a multi-orbit risk assessment interval. Temporally-isolated conjunctions can be analyzed using either BFMC’s CDM mode or SD-VCM mode. Conjunction sequences in which risk accumulates over multiple close events require BFMC’s newly-implemented LD-VCM mode, which is the most computationally intensive of the three.

The repeating conjunction detection threshold value of 10-10 used in this analysis was tested against a subset of 90 CARA high risk conjunctions. Of these, the identification process outlined above properly identified all six conjunctions for which manual processing indicated that collision risk accumulates during multiple encounters. While these initial results are promising, a more detailed analysis of the specific repeating conjunction threshold value is required.

Assessing a Repeating Conjunction

BFMC’s CDM and SD-VCM modes focus on calculating an Pc and an associated Pc confidence interval for a single, temporally-isolated event. BFMC’s newly-implemented LD-VCM mode, on the other hand, assesses risks over an extended, multi-orbit interval. This section outlines the method used to identify if and when the cumulative Pc exceeds a threshold that represents the maximum risk tolerable by a satellite operator.

The original BFMC algorithm subdivided the risk assessment interval into evenly-spaced time bins and counted the number of hits occurring within each interval.7 While this approach worked well for the relatively short duration risk assessment intervals appropriate for temporally-isolated conjunctions, it can become problematic for extended intervals because the algorithm can become complicated and inefficient, due to the extra processing time and memory needed to analyze numerous short-duration bins. For these reasons, the time binning approach used for the original BFMC display software has been replaced by an implementation that determines the cumulative Pc in the form of an empirical cumulative distribution function, based on the exact (not binned) times when hits occur in the simulation. This method works equally well for short-duration and long-duration risk assessment intervals, and eliminates the processing needed to analyze time bins, but retains all of the risk assessment information content produced by the BFMC simulations.

In addition to eliminating the time bins, a burst detection algorithm has been developed to detect groups of closely-spaced hits within the BFMC simulation, summarized as follows. Let Nc be the total number of hits detected by BFMC and tc be the set of hit times relative to τa. The time of the kth hit for 1≤k≤Nhit is denoted as tc(k). The orbital period of an object, P, is determined from the object state8 at epoch. The minimum orbital period is Pmin =minP1,P2. A burst, kburst, is defined as the set of sequential k indexes such that

tck-tck-1≤Pmin9	(6)

In other words, a burst is a set of hit indexes grouped such that the time separating any two adjacent hits does not exceed 1/9th of the minimum orbital period of the two objects. This burst detection algorithm is helpful in identifying repeating conjunctions, and recognizing those cases in which the risk from the individual close approach events effectively blends together in time.

Figure 2: Original BFMC temporal risk plots (left) versus current BFMC plots (right) for a temporally-isolated conjunction between NASA’s Aqua satellite and a debris object. The legend for the current BFMC plots has been omitted for brevity, but contains the same information as the legend above the original BFMC outputs.

Figure 2 shows a comparison of graphs generated by the original BFMC implementation (left panels) and the revised, current version (right panels) for an example temporally-isolated conjunction between two LEO satellites. The top plots show the cumulative Pc as a function of time. The orange dots in the original plots on the left represent center-points of the time bins no longer used by BFMC. The original Pc rate plot (bottom-left) has been replaced by an expanded cumulative Pc plot (bottom-right) that zooms in on the time that the cumulative Pc exceeds a maximum risk threshold. This “Pc threshold crossing” represents an estimate of when the collision risk grows beyond a user-specified, maximum tolerable Pc threshold, set to 10-4 for this example as shown by the dashed black line. The black box inset on the top-right graph indicates the extent of this zoomed view. The new BFMC implementation changes the horizontal time-axis from measuring “Time from TCA” on the left to “Time from Pc Threshold Crossing” on the right. (Alternatively, this axis can plot “Time Since Latest OD Epoch” as will be shown in later examples.) The new plots additionally include the latest OD epoch time (T0), the TCA of the conjunction being analyzed (if temporally-isolated), and the nominal Pc threshold crossing time along with an associated 95% estimation confidence interval. Individual bursts of hits detected in the BFMC simulation are indicated with colored segments across the top of the revised cumulative Pc plot; for this temporally-isolated event, the BFMC system detects only a single burst with a duration indicated by the green segment in the top-right plot of Figure 2. For temporally-isolated conjunctions, the current software can also plot 2D-Pc estimates. Specifically, the horizontal pink lines indicate 2D-Pc values calculated using the Foster and Estes method3 and the blue lines (solid and dashed) indicate 2D-Pc values calculated using the Akella and Alfriend method.22

Example Repeating Conjunctions

The previous section demonstrates how the revised BFMC implementation can display collision risk assessment information for a single, temporally-isolated conjunction. This section demonstrates how information can be displayed for repeating conjunctions, using several examples. As in the previous example, the maximum-risk Pc threshold for all cases has been set to 10-4; this value represents “red” events which often require maneuver planning and execution support by CARA.23

Figure 3: Temporal risk plots for a repeating conjunction between the DIRECTV 12 and DIRECTV 15 GEO cluster satellites using a combined HBR of 10 m.

GEO Repeating Conjunction Example

Figure 3 shows a BFMC risk assessment for a repeating conjunction that occurred in November of 2017 between two closely-spaced, coplanar GEO satellites: DIRECTV 12 and DIRECTV 15. These two active satellites orbit in a tight cluster and experience numerous repeating conjunctions with one another. Figure 3 shows an example 7-day risk assessment for this interaction, beginning at the latest OD epoch time of T0 = 2017-11-07 23:50:43, and calculated assuming a combined hard-body radius of 10 meters (this HBR does not represent the actual combined sizes of these DIRECTV satellites, and is used here only for illustrative purposes). The top panel of Figure 1 shows the nominal separation distance between these GEO satellites over this period, indicating that 14 close approach events occur during this repeating conjunction interaction at a rate of two per orbital period, as mentioned previously. The stair-step pattern of the cumulative Pc curve in the top plot of Figure 3 shows how risk accumulates as the objects approach and recede from one another. BFMC indicates that the cumulative Pc grows during the 7-day risk assessment interval up to a final maximum value of Pc = 9.7310-3 with 95% estimation confidence of 9.3410-3  Pc  10.1410-3. The colored bars along the top panel of Figure 3 show that the BFMC software detects several bursts of hits during the interval. Notably, the first burst spans four days, indicating that the eight repeating conjunctions that occur during this period have such long individual durations that they effectively blend together in time, leading to a rounded stair-step pattern in this portion of the cumulative Pc curve. Subsequent bursts have shorter spans, indicating shorter effective conjunction durations.

The lower panel of Figure 3 shows the area of interest bounding the Pc threshold crossing. For this example, the cumulative Pc exceeds the maximum-risk threshold of 10-4 early during the first day of the risk assessment interval. Specifically, as noted in text on Figure 3, the best-estimate threshold crossing time occurs at 2017-11-08 01:04 (to within a minute), but could conceivably occur about 35 minutes earlier at the 95% confidence estimation level. To avoid collision risk accumulation above the threshold, one of the two GEO satellites would need to perform a risk mitigation maneuver before this time, for this illustrative example.

Figure 4: Temporal risk plots for a repeating conjunction between the DIRECTV 12 and DIRECTV 15 GEO satellites using a combined HBR reduced to 1 m.

Cumulative Pc values and threshold crossing times depend on the primary and secondary epoch states and covariances, as well as the HBR value used in the BFMC calculation. To illustrate this, Figure 4 shows a BFMC risk assessment for the same DIRECTV 12 and DIRECTV 15 repeating conjunction interaction discussed above, but with the hard-body radius reduced in the analysis by an order of magnitude, from 10 m down to 1 m. This HBR reduction has the effect of significantly reducing the cumulative Pc estimates, which reach a maximum of Pc  1.1210-4, about two orders of magnitude smaller than in the previous example. The smaller HBR also has the effect of creating more, better-separated bursts of hits (as marked by the colored bars in the top panel of Figure 4), meaning that the individual conjunctions are less blended in time than in the previous example. Finally, the reduced HBR delays the maximum-risk threshold crossing time by three to five days. Specifically, for this example, the threshold crossing time occurs nominally at 2017-11-13 03:12, but could occur about 33 hours (1.4 days) earlier at the 95% confidence estimation level, as shown in the bottom panel of Figure 4.

LEO Repeating Conjunction

Figure 5 shows a 7-day BFMC risk assessment for a repeating conjunction between non-coplanar LEO satellites that occurred in September of 2017, which differs in nature from the coplanar, closely-spaced GEO examples analyzed above. In this case, BFMC detects three distinct but very brief bursts of hits, marked with the colored dots in the top panel of Figure 5. The middle graph of Figure 1 shows the separation distances between these LEO objects during the risk assessment interval. Note that the close approach events which individually exceed a 2D-Pc threshold of 10-10 (marked with red dots in the middle panel of Figure 1) do not necessarily all result in bursts detected in the BFMC simulation.

Figure 5: Temporal risk plots for a repeating conjunction between two non-coplanar LEO satellites.

The type of repeating conjunction cumulative Pc plot shown in Figure 5 occurs for interactions between objects in orbits with similar periods, but that are not coplanar (these LEO objects have a relative inclination of ~14). Such interactions include short bursts of activity (with durations measured in seconds) interspersed between relatively long periods with no hits or significant accumulation of collision probability. This means that the individual close approaches of the repeating sequence all have short conjunction durations, and are well separated in time (rather than blended or nearly blended as in the previous examples).

Figure 5 also illustrates that Pc can accumulate in bursts of unequal amplitude during repeating conjunction sequences, leading to an uneven stair-step pattern in the cumulative Pc curve. Specifically, the first burst (which occurs at T0+6.29 days) has relatively little effect on the overall Pc, but the second burst (at T0+6.48 days) increases it up to Pc  510-4, well above the maximum risk threshold. The third burst (at T0+6.55 days) increases it further, to a final estimate of Pc  210-3 for the entire 7-day risk assessment period. This highlights the ability of BFMC repeating conjunction analyses to reveal when the most serious encounters occur during multi-orbit risk assessment intervals, in addition to when a maximum tolerable risk threshold will be exceeded.

Conclusions

The analysis yields the following conclusions:

- BFMC’s CDM and BFMC SD-VCM analysis modes can accurately assess risk for short-duration, temporally-isolated conjunctions. However, BFMC’s newly-implemented LD-VCM mode should be used for long-duration and/or repeating conjunction interactions.

- An initial method for identifying repeating conjunctions can be summarized as follows:

  - Propagate the mean SP states and covariances of both objects throughout the multi-orbit risk assessment interval.

  - Find all of the local minima of the primary-to-secondary separation distances for the propagated mean states throughout the risk assessment interval.

  - Calculate 2D-Pc estimates for each of the minima, treating each as an individual temporally-isolated conjunction.

  - Count the number of these 2D-Pc values which exceed a (provisional) repeating conjunction detection threshold of 10-10; if there is more than one, then classify the interaction as a repeating conjunction.

- In addition to the cumulative Pc itself, the maximum-risk Pc threshold crossing time should be used as an additional risk assessment metric for repeating conjunctions. Specifically, to avoid accumulation of collision risk above a maximum tolerable Pc level, a mission must perform a risk mitigation maneuver prior to this threshold crossing time.

This paper presents an update to the BFMC algorithm which allows for the identification and processing of repeating and long-duration conjunctions. An important aspect to repeating conjunction risk assessment is to estimate the time that a maximum-risk Pc threshold is exceeded during the extended, multi-encounter interaction. If this threshold crossing time is not taken into account, a satellite operator analyzing close approach events one at a time may unknowingly delay (or not perform) a risk mitigation maneuver, and put the satellite at elevated risk.

Areas for future research include further tests against repeating conjunctions; the example cases presented in this paper demonstrate the capability of the algorithm but do not rigorously test its overall robustness. Another avenue of research could explore ways to combine multiple, single-encounter 2D-Pc estimates (assuming statistical independence) in order to approximate a consolidated collision probability for a repeating conjunction sequence. Specifically, consolidated 2D-Pc estimates for a large number and variety of repeating conjunction interactions could be compared to the BFMC estimates to assess the effectiveness and limitations of this approach. This consolidated 2D-Pc approach could reduce the computation required for risk analysis significantly by reducing the use of BFMC’s slowest LD-VCM simulation mode. As mentioned previously, more analysis is also needed to determine an optimal Pc threshold level used to identify repeating conjunction events. Finally, the overall challenge remains to identify which conjunctions require BFMC processing, and which specifically require CDM mode, SD-VCM mode, or LD-VCM mode processing.

Symbols and acronyms

af	= the 2nd equinoctial orbital element, component of the eccentricity vector

ag	= the 3rd equinoctial orbital element, component of the eccentricity vector

B	= Ballistic coefficient SP state parameter

D	= collected ensemble of model and environmental data sets required for SP propagation

H	= HBR

i	= index for local minima, i=1…Nm

k	= index for MC collisions or hits, k=1…Nhit

kburst	= set of hit indexes within a single burst

Nc	= the number of MC collisions or hits

Nc(τa,τb)	= the number of MC collisions or hits registered during the time interval τa<t≤τb

Nm	= number of local minima

Ns	= the total number of MC sampling trials conducted in a simulation

n	= the 1st equinoctial orbital element, the mean motion

P	= orbital period

P1	= orbital period for the primary object

P2	= orbital period for the secondary object

Pmin	= minimum orbital period

P	= SP state uncertainty covariance matrix

P0	= SP state uncertainty covariance matrix at the OD epoch time

P1,0	= SP state uncertainty covariance matrix at the OD epoch time for the primary object

P2,0	= SP state uncertainty covariance matrix at the OD epoch time for the secondary object

P1,i	= SP state uncertainty covariance matrix at the local minimum at index i for the primary object

P2,i	= SP state uncertainty covariance matrix at the local minimum at index i for the secondary object

Pc	= collision probability

Pc(τa,τb)	= Pc estimated from an MC simulation for the time interval τa<t≤τb

r	= ECI position vector propagated from OD epoch using SP propagation

r1,i	= ECI position vector propagated to local minimum at index i for the primary object

r2,1	= distance between the primary and secondary objects

r2,1	= rate of change of the distance between the primary and secondary objects

r2,1	= rate of change of the velocity between the primary and secondary objects

r2,i	= ECI position vector propagated to local minimum at index i for the secondary object

S	= Solar radiation pressure SP state parameter

T0	= the latest OD epoch time = max(t1,0,t2,0)

t	= time

t0	= OD epoch time

t1,0	= OD epoch time for the primary object

t2,0	= OD epoch time for the secondary object

tc	= set of times when hits were detected from an MC simulation

tc(k)	= individual hit time at index k

tlmin	= set of times representing the local minima

tlmin,i	= time of the local minimum at index i

v	= ECI velocity vector propagated from OD epoch using SP propagation

v1,i	= ECI velocity vector propagated to local minimum at index i for the primary object

v2,i	= ECI velocity vector propagated to local minimum at index i for the secondary object

X	= SP state vector

X0	= SP state vector at the OD epoch time

X	= mean SP state vector

X0	= mean SP state vector at the OD epoch time

X1,0	= mean SP state vector at the OD epoch time for the primary object

X2,0	= mean SP state vector at the OD epoch time for the secondary object

χ	= the 4th equinoctial orbital element, component of the ascending node vector

λM	= the 6th equinoctial orbital element, the mean longitude

ψ	= the 5th equinoctial orbital element, component of the ascending node vector

τa	= beginning time of a risk assessment interval

τb	= ending time of a risk assessment interval

2D	= two-dimensional

2D-Pc	= 2D collision probability

ASW	= Astrodynamics Support Workstation

BFMC 	= Brute Force Monte Carlo

CARA 	= Conjunction Assessment Risk Analysis

CDM	= Conjunction Data Message

DCA	= Dynamic Calibration Atmosphere

ECI	= Earth-Centered Inertial

HASDM	= High Accuracy Satellite Drag Model

HBR	= Hard Body Radius

LD-VCM	= Long-Duration VCM mode

MC	= Monte Carlo

NASA 	= National Aeronautics and Space Administration

OD 	= Orbit Determination

PDF	= Probability Density Function

SD-VCM	= Short-Duration VCM mode

SP	= Special Perturbations

TCA 	= Time of Closest Approach

VCM	= Vector Covariance Message

References

- L.K. Newman and M.D. Hejduk, “NASA Conjunction Assessment Organizational Approach and the Associated Determination of Screening Volume Sizes,” International CA Risk Assessment Workshop, 19-20 May 2015.

- B.D. Tapley, B.E. Schutz, and G.H. Born, Statistical Orbit Determination, Elsevier Academic Press, Burlington, MA, 2004.

- J.L. Foster and H.S. Estes, “A Parametric Analysis of Orbital Debris Collision Probability and Maneuver Rate for Space Vehicles,” NASA/JSC-25898, Aug. 1992.

- K. Chan, Spacecraft Collision Probability, El Segundo, CA, The AeroSpace Corporation, 2008.

- S. Alfano, “Satellite Conjunction Monte Carlo Analysis,” AAS SpaceFlight Mechanics Meeting, Pittsburgh, PA, Paper 09-233, Feb. 2009.

- B.M. Braun, “The Evolution of Secondary Object Positions in 18SCS Conjunction Data Messages,” AAS Astrodynamics Specialist Conference, Columbia Valley, WA, Paper 17-650, 2017

- D. T.Hall, S.J. Casali, L.C. Johnson, B.B. Skrehart, and L.G. Baaars, “High Fidelity Collision Probabilities Estimated Using Brute Force Monte Carlo Simulations,” AAS Astrodynamics Specialist Conference, Snowbird, UT, Paper 18-244, Aug. 2018.

- D.A. Vallado, Fundamentals of Astrodynamics and Applications, 2rd ed., Microcosm Press, El Segundo CA, 2001.

- C. Sabol, C. Binz, A. Segerman, K. Roe, and P.W. Schumacher, Jr., “Probability of Collisions with Special Perturbations using the Monte Carlo Method,” AIAA/AAS Astrodynamics Specialist Conference, Girdwood, AK, Paper 11-435, Aug. 2011.

- D.T. Hall, M.D. Hejduk, and L.C. Johnson, “Time Dependence of Collision Probabilities During Satellite Conjunctions,” AAS/AIAA Space Flight Mechanics Meeting, San Antonio, TX, Paper 12-271, 2017.

- C. Clopper and E.S. Pearson, “The Use of Confidence or Fiducial Limits Illustrated in the Case of the Binomial,” Biometrika, Vol. 26, pp 404-413, 1934.

- N.L. Johnson, S. Kotz, and A.W. Kemp, Univariate Discrete Distributions, Wiley-Interscience, Hoboken, NJ, 1993.

- B.R. Bowman, W.K. Tobiska, F.A Marcos, C.Y. Huang, C.S. Lin, and W.J. Burke, “A New Empirical Thermospheric Density Model JB2008 Using New Solar and Geomagnetic Indices,” AIAA/AAS Astrodynamics Specialist Conference, Honolulu, HI, Paper AIAA 2008-6348, 2008.

- S.J. Casali and W.N. Barker, “Dynamic Calibration Atmosphere (DCA) for the High Accuracy Satellite Drag Model (HASDM),” AIAA/AAS Astrodynamics Specialist Conference, Monterey, CA, Paper AIAA 2002-4888, 2002.

- Air Force Space Command, “Astrodynamic Standards Software,” http://www.afspc.af.mil, 2018.

- National Research Council of the National Academies, “Continuing Kepler’s Quest Assessing Air Force Space Command’s Astrodynamics Standards,” The National Academies Press, 2012.

- V.T. Coppola, “Evaluating the Short Encounter Assumption of the Probability of Collision Formula,” AAS/AIAA Spaceflight Mechanics Meeting, Charleston, SC, Paper 12-248, Feb. 2012.

- K. Chan, “Hovering Collision Probability,” AAS/AIAA Space Flight Mechanics Meeting, Williamsburg, VA, Paper 15-234, Jan. 2015.

- R.A. Broucke and P.J. Cefola, “On the Equinoctial Orbit Elements,” Celestial Mechanics, Vol. 5, pp. 303-310, 1972.

- D. Vallado, “Covariance Transformation for Satellite Flight Dynamics Operations,” AAS/AIAA Astrodynamics Specialist Conference, Big Sky, MT, Paper 03-526, 2003.

- D. Vallado and S. Alfano, “Updated Analytical Partials for Covariance Transformations and Optimization,” AAS/AIAA Space Flight Mechanics Meeting, Williamsburg, VA, Paper 15-537, 2015.

- M.R. Akella and K.T. Alfriend, “The Probability of Collision Between Space Objects,” Journal of Guidance, Control, and Dynamics, Vol. 23, No. 5, pp. 769-772, 2000.

- L.K. Newman, M.D. Hejduk, and L.C. Johnson, “Operational Implementation of a Pc Uncertainty Construct for Conjunction Assessment Risk Analysis,” AMOS Advanced Maui Optical and Space Surveillance Technologies Conference, Maui, HI, September 2016.

References

Conjunction Assessment Research Analyst, Omitron Inc., 555 E. Pikes Peak Ave, #205  Colorado Springs, CO 80903, USA.

Senior Conjunction Assessment Research Scientist, Omitron Inc., 555 E. Pikes Peak Ave, #205  Colorado Springs, CO 80903, USA.

Chief Astrodynamicist, Omitron Inc., 555 E. Pikes Peak Ave, #205  Colorado Springs CO 80903, USA.